Bush Residential 2 - Diamond Creek

Bush Residential 2 - Eltham

Draft Nillumbik Shire Neighbourhood Character Strategy

71

Bush Residential 2 - Eltham / Eltham North

72

Draft Nillumbik Shire Neighbourhood Character Strategy

Bush Residential 2 - Eltham / Eltham North / Research

Draft Nillumbik Shire Neighbourhood Character Strategy

73

7.5

Character Attributes

Garden Court 1

Character Description

The Garden Court 1 precinct comprises a mix of Modern and Contemporary era building styles. Buildings are generally constructed from brick and have predominantly pitched, tiled roofs. Dwellings are detached, and range between single double storey developments.

Regular and spacious front and side setbacks create a sense of openness in the streetscape. Low or no front fencing allow views to the buildings and front garden spaces. A key characteristic of this precinct is the low-level and formal front gardens, comprising native and non-native species and lawns.

The public realm is characterised by low to medium levels of vegetation, consisting often of grassed nature strips and low level indigenous trees. The lack of front fencing is

notable and allows for the blending of vegetation between the public and private realm.

The streetscape is attributed to a curvilinear subdivision pattern comprising winding streets and court bowls. Roads typically have a wider cross section. Footpaths are generally located on one side of the street and rollover kerbs are common throughout.

Diamond Creek

Greensborough

74

Draft Nillumbik Shire Neighbourhood Character Strategy

Architectural Style, form and layout

- Predominantly Modern and Contemporary era styles constructed in the 1980s and early 2000s.

- Buildings are detached.

- Front façades are articulated with the use of porches, recesses, windows and doorways.

- Building and roof forms are consistent.

Building materials

- Predominantly brick and occasional rendered finish.

Building heights

- 1 to 2 storey single dwellings.

Roof styles

- Predominantly hipped, tiled roofs.

Garages and carports

- Car parking is frequently constructed on the side boundary or in line with the front facade.

Orientation and Siting

- Buildings are oriented parallel to address the street.

- Consistent setbacks across old and new developments between 3m to 6m.

- Side setbacks range from 1 to 3m.

Front Fencing

- A predominant absence of front fences.

- Where a fence is provided it is typically low and solid front fencing, up to 1m in height and constructed of brick.

Gardens and Vegetation

- Low to medium level vegetation, formal front gardens with non-native species.

Public Realm

- Streets are wide and follow a curvilinear, court- based, street pattern with nature strips and rollover kerbs.

- Footpaths are not always present on both sides of the street.

- Lower levels of vegetation with young trees planted on nature strips.

- Grassy nature strips is a common sight.

Yarrambat

Greensborough

Eltham North

North Warrandyte

Research

Diamond Creek

Hurstbridge

St Andrews

Garden Court 1

Legend

Nillumbik municipal boundary Road network

Garden Court 1

CITY OF WHITTLESEA

Panton Hill

Plenty

Wattle Glen

CITY OF BANYULE

Eltham

CITY OF MANNINGHAM

0

2

4km

37 Garden Court 1 Character Area Map

Draft Nillumbik Shire Neighbourhood Character Strategy

75

Neighbourhood Character Objectives

1.

To ensure new development positively responds to the preferred building materials and styles of the precinct, characterised by predominantly low-scale 1-2 storey dwellings with simple building forms and low-pitched roofs with prominent eaves.

To ensure new development follows the topography of the land and does not dominate the streetscape but minimises the dominance of car parking access and structures instead.

To provide spacious front setbacks, that retain the existing rhythm of spacing and allow for established front garden settings.

To maintain and strengthen the presence of established native vegetation, including canopy trees in the front and rear setbacks.

To retain the spaciousness of the streetscape, characterised by absent, or low, front fences with views to front garden areas.

2.

3.

4.

5.

Greensborough

Greensborough

Greensborough

Eltham

76

Draft Nillumbik Shire Neighbourhood Character Strategy

Preferred Character Statement

New development sits within a formal landscape setting and responds to the predominantly low scale, one to two storey dwellings, using consistent materials and simple building forms with low pitched roofs and eaves.

Regular and spacious front setbacks provide space for native plants and canopy trees and understorey vegetation, to mature and reinforce the landscaped

setting. Low level and formal front gardens, comprising native plantings, flow uninterrupted into the public realm.

Garages and carports are often located to the side of the dwellings and are visually unobtrusive and compatible with the development.

Absent or low front fences facilitates the blending of vegetation between the public and private realm, and enable views through to dwellings and front garden areas.

Vistas around street bends and courts are characterised by public spaces featuring native plantings, with a presence of footpaths on either side of the street.

Front Fencing

Gardens and landscaping

Garage storage and vehicle access

Siting and setbacks

Building height and form

Design Guidelines - Garden Court 1

•

New development should complement the 1-2 storey building height, building forms and siting of existing dwellings.

Complement the building forms, materials, colour palette and siting of existing dwellings and articulate the front façades of buildings through the use of setbacks, recesses and windows.

Roofs should provide prominent eaves.

Buildings should be designed to follow the topography of the land, and minimise the need for cut and fill throughout the site.

•

•

•

•

Buildings should be set back a minimum of 1m from one side boundary and 2m from the other side boundary.

Storeys above ground level should be recessed to minimise impact on the streetscape. Orient dwellings parallel to the street.

In the case of multi-dwellings on a lot, design the front dwelling to present as one dwelling to the street through the use of roof forms, materials and design detail.

If more than one dwelling is proposed, provide sufficient separation between each dwelling to allow for the planting of canopy trees and other native vegetation.

•

•

•

•

•

•

Retain established canopy trees and other native vegetation and replant where possible.

Provide large canopy native trees occurring at a density of one to every 200-300m2 across the site.

Prepare a landscape plan to accompany all applications for new dwellings that utilise appropriate native species as identified in the Nillumbik Live Local Plant Local Guide.

•

•

Locate garages and carports behind the line of the front dwelling façade, ensuring that they are integrated with the design of the dwelling.

Provide only one vehicular crossover per typical site frontage.

Limit the width of vehicle accessways and minimise hard paving within the front setback.

Where a side by side development occurs, space the vehicle crossovers to retain the existing rhythm of the street.

Provide landscaping and plantings to the soften the appearance of driveways.

•

•

•

•

•

Fencing should be set back from the front site boundary to allow for landscaping in front of the fence.

A front fence within 3 metres of a street:

- should provide no or a low, open style of up to 0.8m in height or 0.5m in height if constructed in brick.

- may be constructed up to 1.8m when located on a main road where they provide at least 25% permeability.

•

Draft Nillumbik Shire Neighbourhood Character Strategy

77

Character Elements	Design Responses

Zoom in Maps

Garden Court 1 - Greensborough

Garden Court 1 - Greensborough

78

Draft Nillumbik Shire Neighbourhood Character Strategy

Garden Court 1 - Eltham

Draft Nillumbik Shire Neighbourhood Character Strategy

79

7.6

Character Attributes

Garden Court 2

Character Description

The Garden Court 2 precinct comprises predominantly Modern era building styles, with an increasing prevalence of Contemporary infill development. Buildings are generally constructed from brick with pitched, tiled roofs. Dwellings are typically detached, single storey and double storey, with occasional three storey dwellings generally sited on steep hills.

Dwellings and front gardens are clearly visible from the street due to the lack of front fences. Private gardens feature medium to high levels of established native and non-native vegetation, which is generally formally planted and located within generous front setbacks. Landscaping and planting often responds to the slope of the land, with the presence of timber walls, natural stone walls or boulders in the private realm.

The public realm is characterised by a curvilinear street layout, sloping topography, and views to the Plenty River, the Diamond Creek, elevated ridgelines, vegetated areas, tree canopy and Metropolitan Melbourne. Roads typically have a wider cross section that contain nature strips with medium levels of vegetation, consisting often of grassed nature strips with substantial native and non-native canopy trees in some areas. Footpaths are generally located on

one side of the street and drainage consists of both rollover kerbs and traditional kerb and channel layouts.

80

Draft Nillumbik Shire Neighbourhood Character Strategy

Eltham

Greensborough

Architectural Style, form and layout

- Predominantly Modern era styles constructed in the 1980s and early 2000s.

- Buildings are detached.

- Front façades are articulated with the use of porches, recesses, windows and doorways.

- Building and roof forms are consistent.

Building materials

- Predominantly brick and occasional rendered finish across Contemporary development.

Building heights

- Predominantly 1 to 2 storey dwellings and occasional 3 storey dwellings sited on steep topography.

Roof styles

- Predominantly hipped, tiled roofs.

Garages and carports

- Car parking is frequently constructed on the side boundary or in line with the front facade.

- Parking structures and crossovers in the form of double car garages and large, steep driveways.

Orientation and Siting

- Buildings are oriented parallel to address the street.

- Varied setbacks ranging between 3m and 9m.

- Side setbacks range from 2m to 4m.

Front Fencing

- A predominant absence of front fences.

- Where a fence is provided it is typically low and solid front fencing, up to 1m in height and constructed of brick.

Gardens and Vegetation

- Medium to high levels of vegetation, mix of formal and informal gardens with native and non-native species.

Public Realm

- Streets are wide and follow a curvilinear, court- based, street pattern with wide nature strips and rollover kerbs.

- Footpaths not always present on both sides of the street.

- Some areas have substantial native and non- native canopy trees on nature strips.

- Sloping topography.

North Warrandyte

Research

Eltham North

Diamond Creek

Greensborough

Yarrambat

Hurstbridge

St Andrews

Garden Court 2

Legend

Nillumbik municipal boundary Road network

Garden Court 2

CITY OF WHITTLESEA

Panton Hill

Wattle Glen

Plenty

CITY OF BANYULE

Eltham

CITY OF MANNINGHAM

0

2

4km

38 Garden Court 2 Character Area Map

Draft Nillumbik Shire Neighbourhood Character Strategy

81

Neighbourhood Character Objectives

1.

To ensure new development reflects the preferred built form and materials of the precinct, characterised by predominately low to medium scale dwellings with simple building forms and low-pitched roofs.

To maintain and strengthen the bushy garden setting of the precinct by providing spacious front and side setbacks to allow for private landscaping and mix of indigenous and native canopy trees.

To ensure new development does not dominate the streetscape but minimises the dominance of car parking access and structures instead.

To ensure new development provides reasonable sharing of public views to the Plenty River, the Diamond Creek, elevated ridgelines, vegetated areas, tree-canopy and Metropolitan Melbourne.

To retain the spaciousness of the streetscape, characterised by absent, or low, front fences with views to front garden areas which flow into the public realm.

2.

3.

4.

5.

Eltham

82

Draft Nillumbik Shire Neighbourhood Character Strategy

Diamond Creek

Diamond Creek

Greensborough

Preferred Character Statement

Streetscapes are dominated by a mix of indigenous and native canopy trees and vegetation. New development is nestled within sloping landforms and remnant vegetation and is consistent with the predominately low to medium scale built form, using consistent materials and simple building forms with low-pitched roofs and eaves.

Dwellings are visually obstructed and set behind indigenous tree canopy coverage and dense native vegetation. Spacious front and side setbacks provide for the continuation of established vegetation and understorey which responds to the slope of the land.

Garages and carports are often located to the side of the dwellings and are visually unobtrusive and compatible with the development.

Absent or low front fences enable views through to dwellings and lushly vegetated front garden areas which flow uninterrupted into the public realm.

Views to the Plenty River, the Diamond Creek, elevated ridgelines, vegetated areas, tree-canopy and Metropolitan Melbourne are retained.

Vistas around street bends and courts are characterised by nature strips featuring native vegetation and footpaths are on one side the street, enhancing the bushy garden character of the precinct.

Front Fencing

Garage storage and vehicle access

Gardens and landscaping

Siting and setbacks

Building height and form

Design Guidelines - Garden Court 2

•

New development should complement the low to medium scale building height, simple forms and siting of existing dwellings.

Complement the building forms, materials, colour palette and siting of existing dwellings and articulate the front façades of buildings through the use of setbacks, recesses and windows.

Roofs should provide prominent eaves.

Buildings should be designed to follow the topography of the land, and minimise the need for cut and fill throughout the site.

•

•

•

•

•

•

•

•

Set back dwellings at least 3m from one side boundary.

Storeys above ground level should be recessed to minimise impact on the streetscape. Orient dwellings parallel to the street.

Provide at least 30% of the site as permeable surface.

In the case of multi-dwellings on a lot, design the front dwelling to present as one dwelling to the street through the use of roof forms, materials and design detail.

Buildings should be sited to take into account the sharing of views of vegetated areas, tree-canopy coverage, elevated ridgelines and Metropolitan Melbourne.

If more than one dwelling is proposed, provide sufficient separation between each dwelling to allow for the planting of native vegetation and understorey vegetation.

•

•

•

Retain remnant indigenous understorey vegetation and existing native vegetation and trees, where possible and replant where appropriate.

Provide high canopy indigenous and native trees occurring at a density of one to every 200m2 across the site.

Prepare a landscape plan to accompany all applications for new dwellings that utilise appropriate native species as identified in the Nillumbik Live Local Plant Local Guide.

The site area covered by buildings should not exceed 50 per cent.

•

•

•

•

Locate garages and carports behind the line of the front dwelling façade, ensuring that they are integrated with the design of the dwelling.

Where a side by side development occurs, space the vehicle crossovers to retain the existing rhythm of the street.

Minimise hard paving within the front setback.

Provide only one vehicular crossover per typical site frontage.

Provide landscaping and plantings to the soften the appearance of driveways.

•

•

•

•

•

Fencing should be set back from the front site boundary to allow for landscaping in front of the fence.

A front fence within 3 metres of a street:

•

-

should provide no or a low, open style of up to 0.8m in height or 0.5m in height if constructed in brick.

may be constructed up to 1.8m when located on a main road where they provide at least 25% permeability.

-

Draft Nillumbik Shire Neighbourhood Character Strategy

83

Character Elements	Design Responses

Zoom in Maps

Garden Court 2 - Greensborough

Garden Court 2 - Diamond Creek

84

Draft Nillumbik Shire Neighbourhood Character Strategy

Garden Court 2 - Diamond Creek / Wattle Glen

Garden Court 2 - Diamond Creek / Eltham North / Eltham

Draft Nillumbik Shire Neighbourhood Character Strategy

85

Garden Court 2 - Eltham

86

Draft Nillumbik Shire Neighbourhood Character Strategy

7.7

Character Attributes

Garden Court 3

Character Description

The Garden Court 3 precinct comprises Contemporary building styles set within modern subdivisions layouts. Buildings are a mix of detached, single and double storey dwellings and are constructed from brick, render and other contemporary materials. Material finishes consist of muted tones and colours that fit within the landscape setting and roof forms vary. Dwellings are consistently setback from the front and side boundaries and a lack of front fencing creates an open streetscape. Formal garden settings are enhanced by a mix of native and non-native canopy trees, bushes, shrubs and garden beds.

A key characteristic is the intermittent views of surrounding landscapes and vegetation, afforded by curvilinear streets that respond to the undulating topography. Dwellings are often built to take advantage of the landscape, sited on steep hills or sinking below street level, with roof lines at eye level.

Planting in the public realm is generally low-level, with grassed nature strips being a common. Occasional remnant native canopy trees are planted on some streets. Footpaths are generally present and rollover kerbs are common throughout.

Draft Nillumbik Shire Neighbourhood Character Strategy

87

Diamond Creek

Plenty

Architectural Style, form and layout

- Mix of Contemporary architectural styles developed from the 1990s to 2010s.

- Buildings are detached.

- Front façades are articulated with the use of recesses, balconies, porch entrances, windows and doorways.

Building materials

- Predominantly brick or render, and other contemporary materials that are muted in tone.

Building heights

- 1 to 2 storey dwellings.

Roof styles

- Mix of pitched (gable or hip) or flat roofs, constructed of steel or tile.

Garages and carports

- Car parking is frequently constructed in line with the dwelling facade.

Orientation and Siting

- Buildings are oriented parallel to address the street.

- Front setbacks are spacious, ranging from 4m to 9m.

- Side setbacks range from 2m to 4m.

Front Fencing

- A predominant absence of front fences.

- Where fencing is present it is low and incorporates permeable elements or vegetation to soften the appearance.

Gardens and Vegetation

- Low to medium levels of native and non- native vegetation.

Public Realm

- Streets have a modified grid layout, with wide curved roadways with cul-de-sacs.

- Drainage consists of rollover kerbs.

- Footpaths not always present on both sides of the street.

- Street trees feature many native species, are large in scale and irregularly planted.

- Undulating to sloping topography.

Hurstbridge

St Andrews

North Warrandyte

Research

Eltham North

Diamond Creek

Greensborough

Yarrambat

Garden Court 3

Legend

Nillumbik municipal boundary Road network

Garden Court 3

CITY OF WHITTLESEA

Panton Hill

Plenty

Wattle Glen

CITY OF BANYULE

Eltham

CITY OF MANNINGHAM

0

2

4km

39 Garden Court 3 Character Area Map

88

Draft Nillumbik Shire Neighbourhood Character Strategy

Neighbourhood Character Objectives

1.

To ensure new development positively responds to the preferred building materials and styles of the precinct, characterised by predominantly low-scale 1-2 storey dwellings with a Contemporary architectural style that utilises natural materials with muted tones and colours.

To encourage space for increased vegetation planting and native canopy trees within the private realm to enhance the garden setting of dwellings.

To provide for car parking which is in alignment with the dwelling façade and ensure parking does not dominate the streetscape.

To ensure new development provides reasonable sharing of views to the Plenty River, the Diamond Creek, elevated ridgelines, vegetated areas and native canopy trees.

To retain the spaciousness of the streetscape, characterised by absent, low, permeable fences located within dwelling front setbacks.

2.

3.

4.

5.

Diamond Creek

Draft Nillumbik Shire Neighbourhood Character Strategy

89

Diamond Creek

Diamond Creek

Plenty

Preferred Character Statement

New development is sited to take advantage of the views afforded by the undulating landscape and positively responds to the predominantly low scale, one to two storey dwellings, using consistent materials and simple building forms.

New development allows space for increased vegetation and planting of native canopy trees within the front and side setbacks, enhancing the garden setting.

Garages and carports are typically aligned with dwelling façades and are visually unobtrusive and compatible with the openness of streetscapes.

Absent or low front fences enable views through to dwellings and front garden areas, while intermittent views to the Plenty River, the Diamond Creek, elevated ridgelines, vegetated areas and canopy trees are retained.

Vistas around street bends and courts are commonly characterised by grassed nature strips and intermittent remnant native canopy trees, with a presence of footpaths on either side of the street.

Front Fencing

Garage storage and vehicle access

Gardens and landscaping

Siting and setbacks

Building height and form

Design Guidelines - Garden Court 3

•

New development should complement the 1-2 storey building height, simple forms and siting of existing dwellings.

Complement the building forms, natural materials, muted colour palette and siting of existing dwellings and articulate the front façades of buildings through the use of setbacks, recesses and windows.

Buildings should be designed to follow the topography of the land, and minimise the need for cut and fill throughout the site.

Buildings should be designed to follow the contours of the site or step down the site.

•

•

•

•

•

Set back dwellings at least 3m from one side boundary.

Storeys above ground level should be recessed to maintain the appearance of low scale street frontages when viewed from the street

Orient dwellings parallel to the street.

In the case of multi-dwellings on a lot, design the front dwelling to present as one dwelling to the street through the use of roof forms, materials and design detail.

Buildings should be sited to take into account the sharing of views corridors to the Plenty River, the Diamond Creek, elevated ridgelines, vegetated areas and canopy trees.

If more than one dwelling is proposed, provide sufficient separation between each dwelling to allow for the planting of canopy trees and other native vegetation.

•

•

•

•

•

•

Retain established or large canopy trees and other native vegetation where possible.

Provide high canopy native trees occurring at a density of one to every 200-300m2 across the site.

Prepare a landscape plan to accompany all applications for new dwellings that utilise appropriate native species as identified in the Nillumbik Live Local Plant Local Guide.

•

•

Locate garages and carports behind the line of the front dwelling façade, ensuring that they are integrated with the design of the dwelling.

Where a side by side development occurs, space the vehicle crossovers to retain the existing rhythm of the street.

Limit the width of vehicle accessways and minimise hard paving within the front setback.

Paths in the front setback should be integrated with the driveway to maximise space for landscaping and planting.

Provide landscaping and plantings to the soften the appearance of driveways.

•

•

•

•

•

Fencing should be set back from the front site boundary to allow for landscaping in front of the fence.

A front fence within 3 metres of a street:

- should provide no or a low, open style up to 0.8m in height.

- may be constructed up to 1.8m when located on a main road where they provide at least 25% permeability.

•

90

Draft Nillumbik Shire Neighbourhood Character Strategy

Character Elements	Design Responses

Zoom in Maps

Garden Court 3 - Greensborough / Plenty / Diamond Creek

Garden Court 3 - Diamond Creek

Draft Nillumbik Shire Neighbourhood Character Strategy

91

Character Attributes

Garden Residential

7.8

Character Description

The Garden Residential Precinct is characterised by spacious residential areas with consistently sited dwellings in a garden setting. Dwellings have typically been developed in the Post-war era, interspersed by Modern and an increasing prevalence of Contemporary infill development. Buildings are generally constructed of weatherboard and brick and have pitched roofs.

Regular front and side setbacks create a sense of openness in the streetscape. Dwellings and front gardens, comprising of native and non-native vegetation, are clearly visible from the street due to low level gardens and front fences.

Streets have a formal subdivision pattern with modified grid layouts, have a wide cross section which are complemented with paved footpaths on both sides of the street and kerb and channel throughout. Nature strips within this precinct generally contain low to medium levels of vegetation, consisting of grassed nature strips and intermittent indigenous and native canopy tree coverage.

Diamond Creek

Eltham

92

Draft Nillumbik Shire Neighbourhood Character Strategy

Architectural Style, form and layout

- Mix of Postwar, Modern and Contemporary architectural styles constructed of brick or weatherboard.

- Buildings are generally detached with occasional semi-detached styles.

- Front façades are articulated with the use of recesses, balconies, porch entrances, windows and doorways.

Building materials

- Predominantly timber weatherboard or brick.

Building heights

- 1 to 2 storey dwellings.

Roof styles

- Mix of low and high pitched (gable or hip) constructed of metal or tile.

Garages and carports

- Car parking is mixed, located behind the facade of the dwelling or constructed on the side boundary.

Orientation and Siting

- Buildings are oriented parallel to address the street.

- Front setbacks are spacious, ranging from 3m to 9m.

- Side setbacks range from 1m to 3m.

Front Fencing

- Low or transparent front fencing, up to 1m in height and constructed of materials suited to the building.

Gardens and Vegetation

- Well-maintained and low-level established planting, often featuring lawns and a mix of native and non-native planting.

Public Realm

- Streets are wide and follow a formal grid layout with footpaths, nature strips and concrete kerbs and channels.

- Lower levels of vegetation with indigenous and native canopy trees planted on some nature strips. Grassy nature strips is a common sight.

- Flat to sloping topography.

Garden Residential

Legend

Nillumbik municipal boundary Road network

Garden Residential

St Andrews

CITY OF WHITTLESEA

Hurstbridge

Panton Hill

Yarrambat

Wattle Glen

Plenty

Greensborough

Diamond Creek

Eltham North

Research

CITY OF BANYULE

North Warrandyte

Eltham

CITY OF MANNINGHAM

0

2

4km

Draft Nillumbik Shire Neighbourhood Character Strategy

93

Neighbourhood Character Objectives

1.

To ensure new development reflects the preferred built form, characterised by predominately 1-2 storey dwellings with a mixture of architectural styles that utilise natural materials and simple building forms.

To maintain and reinforce the predominant building alignment along the street, characterised by setbacks that contribute to the sense of openness.

To encourage space for increased indigenous and native vegetation planting and canopy trees within the private realm, to strengthen the garden setting of dwellings and contribute to the enhancement of the public realm.

To ensure new development does not dominate the streetscape but minimises the dominance of car parking access and structures instead.

To maintain the openness of the streetscape, characterised by absent front fences.

2.

3.

4.

5.

Eltham

94

Draft Nillumbik Shire Neighbourhood Character Strategy

Diamond Creek

Diamond Creek

Eltham

Preferred Character Statement

New development is sited within the landscape to allow for increased vegetation and native canopy coverage.

Built form reflects the low scale dwellings, using simple building forms with neutral building materials sympathetic to the existing weatherboard and brick dwellings. Buildings are articulated with recesses, porch areas and large windows, to complement the architectural detailing of the older style dwellings.

Consistent front and site setbacks create a sense of openness in the streetscape. New development

allows for space for increased vegetation planting and indigenous and native canopy trees, contributing to the garden character.

Garages and carports are often located to the side of the dwellings and are visually unobtrusive and compatible with the development.

A mix of low scale and permeable front fencing allows for views to dwellings and front garden areas.

Streetscapes are often characterised by public spaces featuring a mix of native and non-native plantings, with a presence of paved footpaths on either side of the street.

Front Fencing

Garage storage and vehicle access

Gardens and landscaping

Siting and setbacks

Building height and form

Design Guidelines - Garden Residential

•

New development should complement the 1-2 storey building height of existing dwellings.

Complement the simple building forms, neutral materials such as brick and weatherboard, and the siting of older, original dwellings.

Articulate the front façades of buildings through the use of recesses, balconies, porch entrances, windows and doorways.

Use contemporary design details that do not mimic older architectural styles. Buildings should provide a pitched roof with prominent eaves.

In cases where eaves are not provided, alternative techniques to articulate the dwelling should be introduced, which may include window shading, window shrouds, and other architectural features.

Buildings should be designed to follow the topography of the land, and minimise the need for cut and fill throughout the site.

•

•

•

•

•

•

•

Buildings should be set back a minimum of 1m from one side boundary and 2m from the other side boundary.

Incorporate upper level setbacks to maintain the appearance of low scale street frontages when viewed from the street.

Orient dwellings parallel to the street.

In the case of multi-dwellings on a lot, design the front dwelling to present as one dwelling to the street through the use of roof forms, materials and design detail.

If more than one dwelling is proposed, provide sufficient separation between each dwelling to allow for the planting of canopy trees and other vegetation.

•

•

•

•

•

Prepare a landscape plan to accompany all applications for new dwellings that utilise appropriate native species as identified in the Nillumbik Live Local Plant Local Guide.

Retain indigenous and native canopy trees and other vegetation where possible.

Provide large canopy indigenous and/or native trees occurring at a density of one to every 200-300m2 across the site.

•

•

•

Locate garages and carports behind the line of the front dwelling façade, ensuring that they are integrated with the design of the dwelling.

Where a side by side development occurs, space the vehicle crossovers to retain the existing rhythm of the street.

Limit the width of vehicle accessways and minimise hard paving within the front setback.

Paths in the front setback should be integrated with the driveway to maximise space for landscaping and planting.

Provide only one vehicular crossover per typical site frontage.

Provide landscaping and plantings to the soften the appearance of driveways.

•

•

•

•

•

•

Fencing should be set back from the front site boundary to allow for landscaping in front of the fence.

A front fence within 3 metres of a street:

- should provide no or a low, visually permeable style up to 1m in height, in an appropriate style for the dwelling.

- may be constructed up to 1.8m when located on a main road where they provide at least 25% permeability.

•

Draft Nillumbik Shire Neighbourhood Character Strategy

95

Character Elements	Design Responses

Zoom in Maps

Garden Residential - Diamond Creek

Garden Residential - Eltham

96

Draft Nillumbik Shire Neighbourhood Character Strategy

Garden Residential - Eltham

Garden Residential - Diamond Creek

Draft Nillumbik Shire Neighbourhood Character Strategy

97

7.9

Character Attributes

Rural Residential 1

Character Description

The Rural Residential 1 precinct is characterised by informal streetscapes and modern residential dwellings. Vegetation cover varies across the precinct, with dwellings set within either low or densely vegetated lots, comprising a mix of native and non-native species. Dwellings are generally single storey with large floor plates, and are constructed of brick and have hipped or pitched roofs.

The public realm is densely vegetated with native remnant trees. Significant front and side setbacks allow for the retention of remnant trees, as well as continued planting with native and non-native species and open lawns.

Buildings are often obscured from view at street level due to density of the private and public realm vegetation.

The streets of this precinct have a largely informal character due to the high levels of roadside vegetation and general lack of public realm infrastructure; often without footpaths and unsealed roads and driveways. Low, post and wire fences delineate the boundary between the private and public realms.

Yarrambat

98

Draft Nillumbik Shire Neighbourhood Character Strategy

Yarrambat

Architectural Style, form and layout

- Predominantly Modern dwellings interspersed by Contemporary infill development, typically constructed of brick.

- Buildings are detached.

- Front façades are articulated with the use of verandahs, windows, doorways and porch entries.

Building materials

- Predominantly brick or rendered finish.

Building heights

- Single storey dwellings with large floor plates.

Roof styles

- Predominantly hipped roof styles, constructed of metal or tile.

Garages and carports

- Car parking is often located behind or in line with the facade of the dwelling.

- Mix of sealed and unsealed, long driveways.

Orientation and Siting

- Inconsistent siting and orientation of dwellings.

- Front setbacks are generous, in excess of 9m.

- Dwellings are separated by spacious side setbacks, in excess of 4m.

Front Fencing

- Low, transparent front fencing up to 1m and often constructed of post and wire.

Gardens and Vegetation

- Planting in front gardens is often a mix of densely vegetated native and non-native species.

- High levels of native vegetation, including remnant trees.

Public Realm

- Informal street planting characterised by high levels of native remnant trees.

- Streets are typically straight and narrow.

- Informal streetscapes typically with no footpaths and no formal drainage infrastructure, usually in the form of trenches or swales.

- Road surfaces vary between sealed and unsealed surfaces.

- Undulating to sloping topography.

Rural Residential 1

Legend

Nillumbik municipal boundary Road network

Rural Residential 1

St Andrews

CITY OF WHITTLESEA

Hurstbridge

Panton Hill

Yarrambat

Wattle Glen

Plenty

Greensborough

Diamond Creek

Eltham North

Research

CITY OF BANYULE

North Warrandyte

Eltham

CITY OF MANNINGHAM

0

2

4km

Draft Nillumbik Shire Neighbourhood Character Strategy

99

Neighbourhood Character Objectives

1.

To ensure new development reflects the preferred built form, characterised by low scale dwellings with a contemporary architectural style that utilises natural materials with muted tones and colours.

To ensure new development does not dominate the wider landscape setting nor penetrates the existing native tree canopy but instead sits behind trees and bushes, often obscured by the vegetation, and minimises site disturbance and the impact of buildings on the landscape.

To strengthen the vegetation-dominated landscape, characterised by large rural lots with generous setbacks that allow retention of native remnant vegetation and continued planting of native trees and gardens.

To ensure new development does not dominate the streetscape but minimises the dominance of car parking access and structures instead.

To retain low, permeable front fences that blend with gardens and existing vegetation that lines the street.

2.

3.

4.

5.

100

Draft Nillumbik Shire Neighbourhood Character Strategy

Yarrambat

Yarrambat

Yarrambat

Yarrambat

Preferred Character Statement

Streetscapes are dominated by large indigenous and native canopy trees with supporting undergrowth. New development is sited to strengthen the vegetation- dominated landscape. Informal bush gardens surround low scale dwellings on spacious lots. New development provides generous front and side setbacks to allow for the retention and continued planting of indigenous and native vegetation and open lawns.

New development reflects the low scale of dwellings, using simple building forms and low pitched roofs. Dwellings do not penetrate the existing remnant native tree canopy and are often obscured by dense roadside vegetation.

Dwellings utilise natural materials with muted tones and colours to fit within the vegetated landscape setting.

The streetscapes feature an informal character which embraces roadside vegetation and a mixture of sealed and unsealed roads.

Garages and carports are hidden from view, often located behind the line of the front dwelling façade and are integrated with the design of the dwelling.

Front fencing is low and permeable, blending with gardens and the unsealed roads.

Front Fencing

Garage storage and vehicle access

Gardens and landscaping

Siting and setbacks

Building height and form

Design Guidelines - Rural Residential 1

•

New development should complement the single storey building height, building forms and large floor plates of existing dwellings.

Articulate the front façades of buildings through the use of verandahs, windows, doorways and porch entries.

Complement the contemporary building forms, natural materials, muted colour palette and siting of existing dwellings.

Buildings should not penetrate the tree canopy, where a canopy is present. Buildings should be designed to follow the contours of the site or step down the site. Buildings should provide a pitched roof with prominent eaves.

•

•

•

•

•

•

Buildings should be set back a minimum of 4m from both side boundaries to enable the planting of indigenous and native trees and understorey planting.

The site area covered by buildings should not exceed 30 per cent. Provide at least 50% of the site as permeable surface.

If more than one dwelling is proposed, provide sufficient separation between each dwelling to allow for the planting of indigenous and native canopy trees and other vegetation.

•

•

•

•

Prepare a landscape plan to accompany all applications for new dwellings that include substantial native trees and species as identified in the Nillumbik Live Local Plant Local Guide.

Retain large, established indigenous and native canopy trees and understorey and provide for the planting of new native trees where possible.

Provide large native canopy trees occurring at a density of one to every 200m2 across the site.

Buildings should be sited and designed to incorporate space for planting of native vegetation.

•

•

•

•

Locate garages and carports behind the line of the front dwelling façade, ensuring that they are integrated with the design of the dwelling.

Provide only one vehicular crossover per typical site frontage.

Minimise paving in front garden areas including driveways and cross overs. Use permeable driveway materials.

Provide landscaping and plantings to the soften the appearance of driveways.

•

•

•

•

•

Fencing should be set back from the front site boundary to allow for landscaping in front of the fence.

A front fence within 3 metres of a street should provide low, open rural style or wire up to

1.2m in height.

•

Draft Nillumbik Shire Neighbourhood Character Strategy

101

Character Elements	Design Responses

Zoom in Maps

Rural Residential 1 - Yarrambat

102

Draft Nillumbik Shire Neighbourhood Character Strategy

Character Attributes

7.10 Rural Residential 2

Character Description

The Rural Residential 2 precinct is characterised by formal streetscapes and a mix of Modern and Contemporary architectural styles. Dwellings have large floor plates and are generally built from brick, or concrete with a rendered finish. Dwellings feature generous front and side setbacks, allowing for landscaping and planting in private gardens.

Dwellings contain varying levels of planting and often feature a mix of native and non-native, strongly textured planting. Open lawns stretching from the dwelling to the street verge are also prominent throughout the precinct. Treatment of batters and slopes is often with walls or similar materials to the dwelling to create flat spaces that relate strongly to the dwelling. Absent, or low and permeable

front fences constructed from timber allow for views through to dwellings and front garden areas.

The streets of this precinct are of a more formal setting in their character; sealed and have a wider cross section

featuring rollover curbs. Footpaths are generally present on one side of the street.

Draft Nillumbik Shire Neighbourhood Character Strategy

103

Plenty

Yarrambat

Architectural Style, form and layout

- Predominantly Modern and Contemporary dwellings developed from the 1990s onwards.

- Buildings are detached.

- Front façades are articulated with the use of verandahs, balconies, windows, doorways and porch entries.

Building materials

- Predominantly brick or rendered finish.

Building heights

- Large, detached 1 to 2 storey dwellings.

Roof styles

- Mix of pitched (gable or hip) or flat roofs constructed of metal or tile.

Garages and carports

- Car parking is often located behind or in line with the facade of the dwelling.

- Driveways are sealed, long and occasionally steep.

Orientation and Siting

- Inconsistent siting and orientation of dwellings.

- Front setbacks are generous, in excess of 9m.

- Dwellings are separated by spacious side setbacks, in excess of 4m.

Front Fencing

- Mix of no or low, transparent front fencing up to 1m and often constructed of timber

Gardens and Vegetation

- Planting in front gardens is often low level and formally planted, with a vast expanses of open lawns.

Public Realm

- Streets have a modified grid layout, with curved roadways with cul-de-sacs.

- Formal streetscapes typically with footpaths and formal drainage infrastructure.

- Low to medium levels of native vegetation, including remnant trees.

- Road surfaces vary between sealed and unsealed surfaces.

- Undulating to sloping topography.

Rural Residential 2

Legend

Nillumbik municipal boundary Road network

Rural Residential 2

St Andrews

CITY OF WHITTLESEA

Hurstbridge

Panton Hill

Yarrambat

Wattle Glen

Plenty

Greensborough

Diamond Creek

Eltham North

Research

CITY OF BANYULE

North Warrandyte

Eltham

CITY OF MANNINGHAM

0

2

4km

104

Draft Nillumbik Shire Neighbourhood Character Strategy

Neighbourhood Character Objectives

1.

To ensure new development reflects the preferred built form, characterised by low scale dwellings with a contemporary architectural style that utilises natural

materials with muted tones and colours and contributes to a semi-rural character.

To ensure new development does not dominate the wider landscape setting but minimises site disturbance and the impact of buildings on the landscape instead.

To strengthen the vegetation-dominated landscape, characterised by large rural lots with significant setbacks that allow continued planting of indigenous and native vegetation and canopy trees.

To ensure new development does not dominate the streetscape but minimises the dominance of car parking access and structures instead.

To retain the spaciousness of the streetscape, characterised by no or low rural style, permeable front fences with views through to landscaping located in the front setbacks of dwellings.

2.

3.

4.

5.

Yarrambat

Plenty

Draft Nillumbik Shire Neighbourhood Character Strategy

105

Plenty

Plenty

Preferred Character Statement

New development is set within the sloping landscape and reflects the semi-rural character of low scale, 1-2 storey dwellings with a contemporary architectural style that utilises natural materials with muted tones and colours.

New development provides significant front and side setbacks to allow for the retention and continued planting of indigenous and native vegetation and canopy trees.

Dwellings are sited on large lots that accommodate a mixture of indigenous and native species, strongly textured planting and vast open lawns.

The streetscapes feature a formal character, with a presence of sealed roads and paved footpaths on either side of the street.

Garages and carports are often located behind the line of the front dwelling façade and are unobtrusive and integrated with the design of the dwelling.

Front fencing is absent or of a low rural style and permeable, allowing for views through to landscaping located in the front setbacks of dwellings.