Overview

4.0	Key Issues and Threats

4.1

In order to inform appropriate future planning provisions for this Neighbourhood Character Strategy (as part of future strategic work), the key character issues applicable to the study area need to be considered.

These threats are summarised below based on the survey findings of the project team, initial consultation with the community and working groups with Council's planning officers.

The following themes have been identified as the key issues and threats:

•

•

•

•

•

•

•

•

•

•

Poor Quality Contemporary Infill Removal of Vegetation & Landscaping Subdivision of Larger Lots

Retirement Villages, Aged Care, Medical & Other Services Dominant Fencing Styles

Siting & Scale

Car Access & Storage Colours & Materials Viewlines

Bushfire Management

The following section provides an overview of each of these issues and threats and identifies areas and locations which are affected by these themes.

36

Draft Nillumbik Shire Neighbourhood Character Strategy

Contemporary Infill

Key Issues and Threats

4.2

Contemporary infill buildings are prominent across much of Nillumbik Shire. These new developments, which feature architectural styles and material palettes that can contrast with existing housing stock, have contributed to a change in character across the study area. In Nillumbik Shire, this type of development has typically occurred in the form of:

Analysis Overview

•

•

replacement houses

additional dwellings or extensions located at the rear of existing lots

redevelopment of sites to accommodate larger residential developments

duplex or side-by-side housing typologies

•

•

The quality of design of infill buildings is varied. Some are respectful of the existing and valued character of a precinct, while others fail to reflect their location and comprise features that disregard the character of existing areas.

Issues associated with contemporary infill include:

•

•

•

•

•

•

Reduced setbacks

Reduced building separation

Vegetation clearing to enable larger scale development Loss of remnant native vegetation/canopy trees Lowered instances of tree retention

Minimal provision of space for deep soil planting and canopy trees

Demolition of existing low density, detached dwellings for larger-scale development

Change in materiality - use of more contemporary materials (concrete, render, etc)

Implications for Guidelines	 Examine the extent to which design objectives can increase setbacks and reduce the bulk and scale of built

form, by:

•

•

•

Reflect minimum side boundary setbacks and minimum landscaping requirements in design guidelines.

Restricting the percentage of the site covered by impermeable surfaces (where relevant).

Strengthen guidelines in regards to deep soil areas and canopy tree/significant vegetation retention in front and rear yards, especially native vegetation.

Reflect typical siting in design guidelines.

Given the existing planning context, there are examples of newer developments in Greensborough, Eltham

and Diamond Creek that are visually bulky, have high site coverage, provide small front and side setbacks, have minimal canopy tree retention (especially native vegetation) and provide limited space for deep soil planting. The introduction of a prominent architectural

element, building form or material not commonly found in the area can incrementally diminish the valued character of the neighbourhood.

The concept of character does not seek to prevent infill development from occurring; rather it aims to ensure that new buildings reflect their location and respect the surrounding built form in a contemporary way. A

contemporary development could successfully respond to its location by reflecting the form and siting of surrounding dwellings.

•

•

•

Draft Nillumbik Shire Neighbourhood Character Strategy

37

Locations affected

- As Nillumbik Shire is characterised by undulating topography and steep slopes, all localities (particularly in some areas in the south) are seeing new volume builds which reflect a 'cut' and 'fill' approach, rather than responding to the slope of the land.

- In locations such as Eltham and Diamond Creek, there has been an increase in larger residential

development and in Greensborough an increased prevalence of two- three storey housing typologies.

- Hurstbridge is seeing higher density, mixed use development of existing allotments.

Other considerations

- Quality of existing dwellings.

- Future population projections.

- Subdivision of larger lots.

Loss of Vegetation and Landscaping

Key Issues and Threats

4.3

Vegetation and landscaping are central elements when determining an area's neighbourhood character. This relates specifically to existing vegetation, including:

Analysis Overview

•

•

•

The public and private realm

The landscaping of individual gardens

The cumulative effect of landscaping across an entire area

The prevalence of canopy coverage

•

Across Nillumbik Shire, vegetation is extensive and is predominately informally planted with canopy trees, native trees and low lying bush/understorey. There also remains remnant native Ecological Vegetation Classes such as remnant Grassy Dry Forest, Valley Grassy Forest, Gully Woodland and other native species.

There is however also a mix of:

•

•

Formal gardens with non-native vegetation

Informally planted gardens and heavily vegetated lots with a mix of native and non-native species

Wide grassy lawns or low-level gardens with limited vegetation

•

A key threat to Nillumbik Shire's neighbourhood character is the loss of vegetation in private gardens, and its replacement with larger developments or non-permeable hard-scaping, such as paving. Additionally, existing landscape character is undermined by contemporary infill builds which do not provide adequate garden space for planting of new vegetation, replanting of native species or canopy trees, which require deep soil and space for roots to grow.

The threat of significant vegetation clearing is particularly relevant in Nillumbik Shire due to State-sanctioned permit exemptions within planning controls applying to a large portion of the Shire, which allow as-of-right removal of vegetation for bushfire management and maintenance.

As most of Nillumbik Shire is within a designated Bushfire Prone Area (BPA), with a portion of the study area covered by the Bushfire Management Overlay (BMO), both

the planning and building systems in Nillumbik Shire comprise of State-sanctioned exemptions which override all other planning triggers for the purposes of bushfire protection including vegetation and landscape protection controls. Details of these exemptions and their impact on neighbourhood character are further discussed Section 5.2 of this Strategy.

Implications for Guidelines

Investigate opportunities to strengthen policy relating to the retention of existing vegetation, by including guidelines which:

•

Strengthen vegetation retention and deep soil areas (where possible).

Require more generous side setbacks in heavily vegetated areas to allow for planting of vegetation and canopy trees.

Apply site coverage maximums to retain sufficient site

•

•

area for vegetation, where relevant.

38

Draft Nillumbik Shire Neighbourhood Character Strategy

Locations affected

Localities under pressure for redevelopment:

- Diamond Creek

- Eltham

- Eltham North

- Hurtsbridge

- North Warrandyte

- Panton Hill

- Research

- St Andrews

- Wattle Glen

Other considerations

- Lack of statutory protection for vegetation.

- Bushfire planning considerations.

- Boundary-to-boundary. development limiting the scope for adequate deep soil areas.

- Moonscaping of lots.

- Excessive site coverage.

- Mandatory garden area. requirements increase areas for planting on lots greater than 400m2, although this does not consider the loss of garden spaces due to paving, swimming pools and tennis courts.

Subdivision

Key Issues and Threats

4.4

Subdivision of lots within Nillumbik Shire is particularly prevalent in localities that are not within an LDRZ or covered by DDO2 and DDO3 which specify a minimum lot size. Subdivision is often associated with mass vegetation removal in order to maximise the development yield.

The subdivision of lots throughout Nillumbik Shire has a direct impact on neighbourhood character due to:

Analysis Overview

•

•

•

•

•

•

Reduced setbacks

Reduced building separation

Vegetation clearing to enable larger scale development Loss of remnant native vegetation/canopy trees Lowered instances of tree retention

Minimal provision of space for deep soil planting and canopy trees

Demolition of existing low density, detached dwellings for larger-scale development

Change in materiality - use of more contemporary materials (concrete, render, etc)

•

Considerations

•

Consider minimum subdivision sizes in areas that are heavily vegetated and where necessary - to be

considered in conjunction with recommendations of a future Housing Strategy and Residential Development Framework.

•

A key threat to neighbourhood character in many localities is the subdivision of lots and the resulting incremental change in the type of housing stock as well as ineffective integration of the natural and urban environments.

Subdivision must be done in a manner that is contextually sensitive to the distinct neighbourhood character of each locality in Nillumbik Shire.

23 Subdivision with duplex housing development - Diamond Creek

Draft Nillumbik Shire Neighbourhood Character Strategy

39

Locations affected

- Areas not covered by a DDO or LDRZ.

- Yarrambat is covered by DDO2, which is constrained to controlling subdivision size with limited density and setback provisions.

Other considerations

- Lack of statutory protection for vegetation.

- Future demographic considerations.

- Choice of housing typologies.

Larger Scale Development - Retirement Villages, Aged Care, Medical & other Services

Key Issues and Threats

4.5

There is potential for an increased pressure for mixed-use development in residential areas throughout Nillumbik Shire. This type of development includes:

Analysis Overview

•

•

•

•

Retirement villages Aged care facilities Medical centres

Childcare centres

Clause 53.17 Residential Aged Care facility allows for an aged care facility to have a maximum building height of 16 metres on land located within the General Residential Zone (GRZ), Mixed Use Zone (MUZ), Neighbourhood Residential Zone (NRZ), Residential Growth Zone (RGZ) or Township Zone (TZ).

The increasing pressure for these services to be developed in residential areas covered by the following zones, may impact upon existing character in Nillumbik Shire.

For example, increased pressure to develop Aged Care facilities that are of a greater scale and form than

surrounding residential dwellings, will impact the rhythm of existing streets and the ability to provide planting of canopy trees or the retention of significant vegetation, directly impacting upon neighbourhood character.

Implications for Guidelines & Considerations

•

Consider guidelines which prioritise the retention of significant vegetation and the provision of ample space for canopy trees.

Consider impact of upper level setbacks to existing residential uses.

•

24 VMCH St Thomas' Retirement Village - Greensborough

40

Draft Nillumbik Shire Neighbourhood Character Strategy

Locations affected

- Localities covered by GRZ (Diamond Creek, Eltham, Research and Greensborough).

- Areas under pressure for redevelopment (e.g. areas with lower quality housing stock, in close proximity to activity centres).

Other considerations

- Whilst the increased emergence of medical and childcare centres in residential areas is a present issue within the Shire, the development

of retirement villages and aged care facilities is a potential future threat.

- Future demographic considerations, including an ageing population, may require aged care or retirement facilities in the future.

- Choice of housing typologies.

Dominant Fencing Styles

Key Issues and Threats

4.6

The treatment of front boundaries varies throughout Nillumbik Shire, with many residential areas characterised by no front fencing or low fences that allow views to the

front garden or dwelling, use vegetation as a border treatment or have open frontages. The combined effect of these boundary characteristics is open streetscapes and a vegetated dominated atmosphere where fences are a less prominent feature.

As the majority of areas within Nillumbik Shire are characterised by no or low front fencing, the introduction of more dominant fencing styles that block views to gardens and dwellings would directly impact existing character.

High, solid fences would undermine the blending of vegetation in the public and private realm, which is a key characteristic of Nillumbik Shire's residential areas. High, solid front fences are starting to emerge in areas under pressure for redevelopment, as well as areas with frontages to major roads or smaller roads of high usage.

Analysis Overview

Implications for Guidelines

•

Strengthening zone and overlay schedules that relate to front fence design ensuring they are appropriate to local neighbourhood character, including objectives that:

-

encourage fencing that reflects the typical styles in the area.

-

encourage a lack of front fencing in areas where this is common.

discourage the development of high, solid fences that limit visual permeability.

consider only allowing higher front fences on main roads where there is particular need to provide privacy and mitigate against noise in the front setback.

consider guidelines that locate front fencing within a site boundary to allow space for plantings in front of the fence.

25 Streetscape with low and transparent timber fencing - Plenty

-

-

-

•

Resist pressure from applications to incorporate high front fences in order to utilise the area as secluded private open space.

26 Residential streetscape with solid fencing - Diamond Creek

Draft Nillumbik Shire Neighbourhood Character Strategy

41

Locations affected

- Areas under pressure for redevelopment.

- Areas with frontages to main roads.

- However it is noted that tall or solid fencing may impact the character in all localities.

Other considerations

- A front fence includes any fence within 3 metres of the street and should not exceed the maximum fence height.

- Noise impacts and privacy concerns from the public realm.

- Security considerations.

Inconsistent Siting & Scale

Key Issues and Threats

4.7

Siting and scale are central elements of determining an area's neighbourhood character. Topography is a

distinguishing characteristic of Nillumbik Shire, ranging from gentle undulating slopes to very steep areas along waterways and ridgelines. Subsequently, dwellings are often built to take advantage of the landscape and to

enable views of ranges, valleys and Metropolitan Melbourne (e.g. split level designs). Notably, dwellings in the northern residential areas are typically sited on steep hills or below street level.

In terms of character, it is important that new developments differ in detailed design elements, whilst simultaneously respecting the form, siting, scale and vegetation coverage of existing dwellings in a streetscape or character area. In Nillumbik Shire, it is critical that new development respects the varying topographic conditions, limiting development located on ridgelines and the penetration of built form above tree canopy coverage.

The scale and siting of dwellings on their residential lot has a significant influence on the character of a streetscape. In areas with a consistent neighbourhood character, dwellings will generally follow a typical pattern with the rest of the streetscape and will conform to similar building envelopes.

New developments with substantially reduced setbacks interrupt the established pattern of streets and increases the built form enclosure of the street. As Nillumbik Shire is predominately characterised by large allotments that follow a curvilinear subdivision layout, the siting of new

buildings on irregular-shaped lots may also disrupt existing streetscape rhythm.

Analysis Overview

Implications for Guidelines

- Examine best-practice examples of maintaining views between dwellings, planting in rear gardens, creating a sense of spaciousness and streetscape rhythm, by including guidelines that:

-

encourage upper level setbacks and front and side setbacks at ground level to facilitate the provision of landscaping and significant vegetation.

maintain key siting considerations such as dwelling orientation and side setbacks.

specify preferred roof forms where this is consistent.

-

-

27 Dwellings sited on steep hill - Diamond Creek

42

Draft Nillumbik Shire Neighbourhood Character Strategy

Locations affected

- Majority of the residential areas within Nillumbik Shire are characterised by undulating slopes, and some localities feature steep areas along waterways and ridgelines.

- Areas under pressure for redevelopment, subdivision and contemporary infill development (e.g. areas in close proximity to activity centres).

Other considerations

- Redevelopment/extensions of single dwellings into larger scale and medium density housing.

- Development of aged care, retirement, medical centres and child care facilities within residential areas.

- Within the BMO, replacement or extended dwellings require a

Bushfire Management Plan and the creation of a defendable space. As a result, developments characterised by a large building footprint will see high levels of vegetation removal.

Dominant Car Access & Storage

Key Issues and Threats

4.8

The siting and design of car access and storage has a direct impact on the character of streetscapes. Additional or widened crossovers result in increased hard paving and loss of nature strip planting. Reduced garden space and permeable ground for sustaining vegetation also occurs due to hard paving areas within front setbacks for car parking or other purposes. Landscaping along driveways to soften this impact is often non-existent or too narrow.

Car parking structures vary greatly across the Nillumbik Shire. Car parking structures can dominate the frontage of a property in two ways. Firstly, by being located in line with or forward of the dwelling. Secondly, by occupying a large proportion of the frontage, for example by the use of a triple or double garage. Both of these car parking structure types are present in Nillumbik Shire. As a result, streets

are less attractive for pedestrians, passive surveillance is reduced, and the positive aspects of neighbourhood character in the streetscape are undermined.

In order to address this, dominant car parking structures should be discouraged. Guidelines should also seek to minimise the hard surfaces in front setbacks and reduce the crossovers to one per property.

Landscaped strips should be encouraged along driveways located along property boundaries. Landscaping works to break up expanses of hard, impervious surfaces, particularly where driveways abut across property boundaries.

28 Dwellings with large car storage areas and expanses of hard surfaces

Analysis Overview

Implications for Guidelines

- Where applicable, guidelines should be incorporated in provisions in the Nillumbik Shire Planning Scheme that:

-

Limit the proportion of the site covered by impermeable surfaces, particularly in front setbacks.

Locate garages and carports behind the front facade of the dwelling.

Provide only one vehicular crossover per typical site frontage.

-

-

Draft Nillumbik Shire Neighbourhood Character Strategy

43

Locations affected

- Areas under pressure for redevelopment, subdivision and contemporary infill development (e.g. areas in close proximity to activity centres).

Other considerations

- Infill of additional dwellings at the rear of the lot.

Inconsistent Colours & Materials

Key Issues and Threats

4.9

In Nillumbik Shire, the consistency of colours and materials is a defining aspect of the character of some localities. However, the colour palettes and materiality of contemporary design schemes of new builds in some localities has caused built form to stand out from their surroundings, rather than blending in within them.

Some areas of Nillumbik Shire have an identified and recognisable character due to the consistency of a muted and earthy toned colour palette and natural building materials. For example, Eltham and North Warrandyte feature notable concentrations of mud brick dwellings characterised by muted tones and natural material finishes which complements Nillumbik Shire's 'leafy' character feel and landscaped setting.

The majority of dwellings in Nillumbik Shire are constructed of varied materials, ranging from brick and concrete

render combinations, timber weatherboard and other contemporary materials. However, many areas are often characterised by muted and earthy hues on both façades and roofs. However, much of the contemporary development makes use of a grey-based palette, this exaggerates the contrast in building style and form

between new and old. Sometimes, adopting the same (or similar) materials and finishes can be a decisive factor in allowing a contemporary design to complement, rather than compete, with its surroundings.

This Strategy seeks to clearly identify the areas where maintaining similar or complementary surface finishes is fundamental to the preferred neighbourhood character.

Analysis Overview

Implications for Guidelines

- Identify areas in which a specific range of material finishes and colour palette should remain a key part of the preferred future character.

29 Contemporary dwelling featuring varied material finishes - Yarrambat

30 Mud brick residential dwelling - Eltham

44

Draft Nillumbik Shire Neighbourhood Character Strategy

Locations affected

- Diamond Creek and Plenty- built form outcomes emerging as a result of DPO objectives.

- Areas under pressure for redevelopment (e.g. areas with lower quality housing stock, in close proximity to activity centres).

- Diamond Creek and part of Eltham North - areas that have no SLO coverage; no permit triggers for single dwellings.

- However it is noted that contemporary development may disrupt the consistency of

earthy colours and materials in all localities.

Other considerations

- Redevelopment/extensions of single detached dwellings into medium density housing.

- Development of aged care, retirement, medical centres and child care facilities within residential areas.

- Sustainability factors.

Key Issues and Threats

4.10 Interrupted Viewlines

In Nillumbik Shire, maintaining the views from the public realm between dwellings and vistas across the landscape is a critical component of neighbourhood character.

A significant element of character across multiple localities within Nillumbik Shire is the undulating topography, which affords picturesque views of vegetated areas and elevated ridge lines.

Viewlines to the tops of tree canopies and ridgelines is also a key feature of neighbourhood character within many

of the bushy and 'leafy' vegetated areas. An important consideration for new development is to ensure it does not visually penetrate these viewlines.

However, as mentioned previously, contemporary infill development typically utilises a 'cut' and 'fill' method to accommodate volume builds which do not effectively respond to the slope of the land. Although this allows for the continuation of shared views, it disrupts the street rhythm and reduces vegetation retention.

Analysis Overview

Implications for Guidelines

•

Consider guidelines for the identification of and reasonable protection of shared viewlines, including:

-

-

-

-

Upper level setbacks

Front and side setbacks at ground level Siting

Building bulk

•

Consider objectives and guidelines to ensure new development does not visually penetrate tree canopies and/or ridgelines, where this is a key feature of an areas character.

31 Residential area with rural views - Yarrambat

Draft Nillumbik Shire Neighbourhood Character Strategy

45

Locations affected

- Areas located on steep slopes or within close proximity to waterways.

- Areas with long ranging views to Diamond Creek, Plenty River, Yarra River, vegetated areas, tree-canopy elevated ridge lines, rolling hills, Metropolitan Melbourne etc.

- However it is noted that new development may disrupt viewlines in all localities characterised by undulating topography.

Other considerations

- Development of aged care, retirement, medical centres and child care facilities within residential areas.

- Choice of housing typologies.

VCAT Cases Summary

5.0	Performance of Existing Settings

5.1

The following is a summary of a selection of VCAT cases relating to neighbourhood character in Nillumbik Shire.

The key implications for this Strategy are outlined below.

Effectiveness of Existing Controls

Of the VCAT cases reviewed, the majority indicate that the existing planning controls that are used to implement neighbourhood character objectives are performing relatively well.

The Significant Landscape Overlay (SLO3) was cited in six cases as having provided an effective decision making

framework for VCAT members, resulting in either an affirmation of Council's prior permit refusal, or, approval of an amended permit. In the case of an amended permit, VCAT required additional design changes including increased setbacks and space for landscaping to achieve the objectives of SLO3.

Where a case involved an existing DDO (DDO2), Council's concerns relating to building height and form were

not upheld, with VCAT ruling that proposed multi level dwellings were in accordance with the objectives of DDO2, which is limited to controlling subdivision size. The design response to the character area was deemed to be appropriate as sufficient setbacks and space for landscaping were provided to maintain the bush setting. This suggests that there may be scope to revise existing

DDOs, or implement new controls with guidelines targeted at generating built form outcomes that respond to identified character attributes.

Gaps in Controls

Two VCAT cases reviewed, concerned proposed development in residential locations that were not subject to either an SLO or a DDO; Council's main tools for implementing its Neighbourhood Character Guidelines.

In these instances Council raised concerns with built form elements including inadequate setbacks, excessive visual bulk and limited opportunity for retention of vegetation or replacement landscaping.

However, VCAT were satisfied that the developments met the neighbourhood character objectives of Clause 55 (Rescode) and were therefore deemed appropriate for approval.

While the relevant neighbourhood character objectives were outlined within local policy at Clause 22.12 (now 15.01-5L), greater weight was placed on the objectives of Clause 55. This highlights that in the case of achieving neighbourhood character objectives, it is important that they are implemented by utilising Clause 55 variations within the zone schedule, or through overlays such as an SLO to ensure that they are given appropriate statutory weighting.

Case

Calzone v Nillumbik SC

Hopping v Nillumbik SC

Merrigan v Nillumbik SC

Bible Street Project Pty Ltd v Nillumbik SC & Ors

Sam Battiato for Creative Design Studio v Nillumbik SC & Or

Architectural Home Design v Nillumbik SC

Sarantis v Nillumbik SC

Thomson v Nillumbik SC

Table 3 – Reviewed cases that identify effectiveness of existing controls

Case

Glenrich Investments Pty Ltd v Nillumbik CC

Scheiber & Co Pty Ltd v Nillumbik SC

Table 4 – Reviewed cases that identify gaps in existing controls

46

Draft Nillumbik Shire Neighbourhood Character Strategy

Bushfire Management

5.2

The majority of Nillumbik Shire is a Designated Bushfire Prone Area (BPA) under section 192A of the Building Act 1993. Also, a large proportion of land within the Shire

is subject to Bushfire Management Overlay (BMO). While allowing residents to mitigate bushfire risk on their properties is integral to the protection of human life, anecdotal evidence suggests that an unfortunate consequence is the impact on neighbourhood character. The cumulative impact of vegetation removal from individual properties poses a threat to the highly valued ‘leafy green’ character of the Nillumbik Shire.

The importance of vegetation to Nillumbik Shire's character and biodiversity is recognised through various parts of the Nillumbik Shire Planning Scheme, as noted in previous sections. The neighbourhood character analysis has

also identified vegetation as a critical component of the character of majority of localities, and contributes overall to Nillumbik Shire's character.

Exemptions apply to buildings built or approved prior to 10 September 2009 and are set out in Clause 52.12 (Bushfire Protection: Exemptions). The exemptions apply regardless of whether a permit is required to remove vegetation under any other provision of the planning scheme

(e.g. Clause 52.17: Native Vegetation Protection Overlay, Environmental Significance Overlay or the like). Specifically, the exemptions override all other planning permit triggers, meaning that Council does not have the power to prevent the removal of vegetation covered by the exemptions.

This Neighbourhood Character Strategy does not have the capacity to influence, alter or change the exemptions that are specified under Clause 52.12.

Analysis undertaken as apart of this Strategy has highlighted the community's concern about mass vegetation removal and the associated impact on the valued bushy character of localities. Part of this concern may arise from a question of whether any property owners are taking advantage of bushfire exemptions to maximise development yield, rather than simply to mitigate a genuine bushfire risk.

33 Densely vegetated rural road - Yarrambat

Draft Nillumbik Shire Neighbourhood Character Strategy

47

34 Bushfire Management Overlay Map

48

Draft Nillumbik Shire Neighbourhood Character Strategy

Bushfire Management

Legend

MURRINDINDI

Nillumbik municipal boundary	SHIRE COUNCIL

Road network Study Area

Bushfire Prone Area (BPA)

Bushfire Management Overlay (BMO)

CITY OF WHITTLESEA

St Andrews

Hurstbridge

Yarrambat

Panton Hill

Plenty

Wattle Glen

Diamond Creek

Greensborough

Eltham North

Research	YARRA RANGES

SHIRE

Eltham	North Warrandyte

CITY OF BANYULE

0	2.5	5km

CITY OF

MANNINGHAM

Summary of Draft Neighbourhood Character Areas

Draft Neighbourhood Character Areas

6.0

This section outlines the draft neighbourhood character areas identified within Nillumbik Shire as part of the desktop analysis and site surveys undertaken for this Strategy.

These draft neighbourhood character areas also reflect the key values identified through initial community consultation undertaken in April and May 2022.

The draft character areas will replace the character areas identified as part of Nillumbik Shire’s previous Neighbourhood Character Study and Design Guidelines (amended in 2003).

The table below provides an overview of the draft character areas identified by this Strategy and also demonstrates the key differences between each character type/area.

Map 35 depicts the draft character areas based on the criteria outlined in the table.

Draft Nillumbik Shire Neighbourhood Character Strategy

49

Bush Residential

Defined by heavily vegetated residential areas generally consistently sited dwellings in a bush setting and sometimes informal street pattern; generally modified grid and curvilinear style streets.

Bush Residential 1

- Mix of post-war, modern and contemporary dwellings

- Predominantly gable roof forms

- High levels of native vegetation, screening dwellings from view

- Presence of unsealed, dirt or gravel roads

- Often no footpaths on either side of road

Bush Residential 2

- Predominantly post-war and modern dwellings

- Mix of gable, hipped and flat roof forms

- Formally landscaped gardens

- Predominantly sealed, asphalt streets

- Typically footpaths on at least one side of road

Garden Court

Features spacious residential areas with sometimes informally sited dwellings in a garden setting and curvilinear street layout (courts / cul-de-sac), with a mix of flat and undulating topography.

Garden Court 1

- Modern and contemporary dwellings

- Low to medium levels of vegetation

- Predominantly flat topography

Garden Court 2

- Predominantly modern dwellings

- Medium to high levels of vegetation

- Sloping to steep topography

Garden Court 3

- Contemporary dwellings

- Low levels of vegetation

- Sloping to steep topography

Summary of Draft Neighbourhood Character Areas

50

Draft Nillumbik Shire Neighbourhood Character Strategy

Garden Residential

Characterised by spacious residential areas with consistently sited dwellings in a garden setting and gridded street layout. Dwellings have typically been developed in the post-war era, interspersed by modern dwelling types and an increasing prevalence of contemporary infill development.

Rural Residential

Typically consists of contemporary and modern dwellings on large residential lots, set within semi-rural areas of the Shire. Dwellings feature large footprints and are often inconsistently sited on curvilinear street patterns.

Rural Residential 1

- Informal streetscapes

- Densely vegetated public realm

- Significant front setbacks

Rural Residential 2

- Sealed, asphalt streets

- Low levels of vegetation

- Formal gardens, expansive lawns

Note: A series of zoomed in locality maps are provided in the Appendix.

35 Identified Draft Neighbourhood Character Areas

Draft Nillumbik Shire Neighbourhood Character Strategy

51

What is preferred neighbourhood character?

7.0	Neighbourhood Character Precinct Profiles

7.1

As outlined in Planning Practice Note 90, under clause 54 and clause 55, new development should respect the existing neighbourhood character or contribute to a preferred neighbourhood character.

Preferred neighbourhood character is either:

- the existing character of an area; or

- an identified future neighbourhood character different from the existing character of an area.

Where the existing neighbourhood character is the preferred neighbourhood character, it is important to identify the existing features and characteristics of the area to be respected.

A preferred neighbourhood character statement can articulate the valued features and characteristics of an area to be respected by new development. There is no prescribed format for a preferred neighbourhood character statement. Its form will depend on several factors including the features and characteristics of an area or municipality, the housing outcomes sought, and the views of the local community.

It is important that preferred neighbourhood character statements are ‘forward-looking’ so that if an area is identified for increased housing growth, the growth is not undermined by neighbourhood character policies that seek to maintain the existing neighbourhood character.

36 Dwelling with densely vegetated gardened areas - Diamond Creek

52

Draft Nillumbik Shire Neighbourhood Character Strategy

How to read the Character Precinct Profiles

7.2

Neighbourhood character precinct profiles have been prepared for each neighbourhood character area identified within Nillumbik Shire. Each profile provides a summary

of character area with associated character objectives and design guidelines to provide guidance to future development, ensuring that it reflects the preferred character as best as possible.

The profiles are intended to replace the character areas identified as part of Nillumbik Shire’s previous

Neighbourhood Character Study and Design Guidelines (amended in 2003).

The precinct profiles include:

•

•

•

•

•

•

•

Character Description; A Character Area map;

A summary of key character attributes Preferred Character Statement; Preferred Character Objectives; Design Guidelines; and

Photos

Neighbourhood Character Objectives and Design Guidelines

The purpose of the character objectives and design guidelines is to provide clear direction and guidance on built form outcomes based on preferred neighbourhood character outcomes.

In alignment with Planning Practice Note 91, the Preferred Character Statement directly informs the five design objectives per character area, which can be specified in a schedule to a residential zone to implement the preferred neighbourhood character.

Effective design guidelines should be used as a basis for Council planners when assessing planning applications. Design guidelines that are to be translated into a zone schedule, to vary ResCode standards, should be enhanced by including specific details of the schedule changes (i.e. permeability, site coverage, front and side setbacks).

Draft Nillumbik Shire Neighbourhood Character Strategy

53

Character Attributes

Architectural Style, form and layout

- Era of buildings

- Existing building quality

- Scale of existing development

- Dominant residential form

- Dominant residential typology

Building Materials

- Primary materials

Building Heights

Roof Styles

- Roof form

- Roof material

Garages and Carports

- Dominance of parking structures

Orientation and Siting

- Front setback

- Side setback

- Consistency of setbacks

Front Fencing

- Front fence height

- Front fence type

Gardens and Vegetation

- Private landscaping

- Private garden type

Public Realm

- Public landscaping

- Street tree types

- Footpath presence

- Road layout

- Road surface

- Views

- Topography

Character Attributes

Bush Residential 1

7.3

Character Description

The Bush Residential 1 precinct is characterised by informal streetscapes and dense vegetation in both the public and private realms. Dwellings are predominantly Modern and Contemporary in their design, with occasional examples of Post-war architecture. Material finishes consist of muted tones and colours that fit within the landscape setting and dwellings are constructed in varied materials, including mudbrick.

Significant setbacks from the street level contribute to the heavily vegetated character of the area, affording ample space for the retention and planting of indigenous, native and some non-native vegetation. Dwellings are sited within lots that are also heavily vegetated with native, established gardens that appear visually unified with significant indigenous tree canopy and Australian native vegetation

in the public realm. Dwellings are often obscured from view at street level due to the density of the planting. Low, transparent front fences often constructed of post and wire materials delineate the boundary between the private and public realms.

Road and vehicle crossovers are generally unsealed, with swale drains and often no footpaths. Typically roads are narrower than standard roads.

Eltham North

Eltham North

54

Draft Nillumbik Shire Neighbourhood Character Strategy

Architectural Style, form and layout

- Mix of Post-war, Modern and Contemporary era styles, reflective of varying building styles and material finishes.

- Buildings are detached.

- Front façades are articulated with the use of balconies, porch entrances, windows and doorways.

Building materials

- Predominantly brick or render, and other contemporary materials.

Building heights

- 1 to 2 storey dwellings.

Roof styles

- Predominantly gable fronted roof styles, constructed of metal.

Garages and carports

- Car parking is often located behind the facade of the dwelling or constructed on the side boundary.

- Long, steep driveways that are typically unsealed.

Orientation and Siting

- Inconsistent siting and orientation of dwellings.

- Front setbacks are generous, in excess of 9m.

- Dwellings are separated by spacious side setbacks, in excess of 4m.

Front Fencing

- Mix of no or low and transparent front fencing up to 1m and often constructed of post and wire.

Gardens and Vegetation

- High levels of vegetation, specifically indigenous, native and some non-native species, including remnant trees that visually unite the public and private realms and often screen dwellings from view.

Public Realm

- Streets have a modified grid layout, with curved roadways with cul-de-sacs.

- Informal street planting characterised by high levels of native remnant trees.

- Informal streetscapes typically with no footpaths and narrow unsealed roads and vehicle crossovers, with the surface consisting of gravel or dirt.

- Drainage usually consist of trenches or swales, with individual water pipes located under driveways.

- Undulating to sloping topography.

Bush Residential 1

Legend

Nillumbik municipal boundary Road network

Bush Residential 1

St Andrews

CITY OF WHITTLESEA

Hurstbridge

Panton Hill

Yarrambat

Plenty

Wattle Glen

Greensborough

Diamond Creek

Eltham North

Research

CITY OF BANYULE

North Warrandyte

Eltham

CITY OF MANNINGHAM

0

2

4km

Draft Nillumbik Shire Neighbourhood Character Strategy

55

Neighbourhood Character Objectives

1.

To ensure new development reflects the preferred built form, characterised by predominantly 1-2 storey dwellings with varied architectural style that utilises natural materials with muted tones and 'earth' colours to fit within the heavily vegetated landscape.

To maintain and enhance views corridors to the Yarra River, the Diamond Creek, the Watsons Creek, elevated ridgelines, and the vistas, streetscapes, and backdrops of the precinct, characterised by large native canopy trees and lush understoreys of shrubs and grasses.

To ensure new development sits within the existing indigenous and native tree canopy, minimising the dominance of car parking access and structures.

To maintain and strengthen the spaciousness and bush setting surrounding dwellings, minimising disruption to existing indigenous and native tree and plant species.

To retain the bushy informal transition of public and private realms, characterised by absent or permeable low front fences.

2.

3.

4.

5.

56

Draft Nillumbik Shire Neighbourhood Character Strategy

St Andrews

Panton Hill

Eltham

North Warrandyte

Preferred Character Statement

Streetscapes are dominated by large indigenous and native canopy trees with supporting undergrowth.

Dwellings are sited to sit within the undulating landscape and to integrate with the informal style of the indigenous and native vegetation which flourishes in a spacious bushy setting.

View corridors to the Yarra River, the Diamond Creek, the Watsons Creek, elevated ridgelines, vegetated areas and significant canopy trees are retained. Often formalised footpaths are not present. Mostly roads are unsealed and wind informally through the trees with minimal delineation between public and private realms.

New development positively responds to the predominantly low scale, one to two storey dwellings, using consistent natural, earthy 'coloured' materials and simple building forms to fit within the heavily vegetated setting.

Generous front and side setbacks provide for generous indigenous and native vegetation which embrace remnant trees, and screen dwellings from view.

Garages and carports are hidden from view, often located behind the line of the front dwelling façade and are integrated with the design of the dwelling.

Absent, low or transparent, front fencing contributes to the bushy and informal transition between public and private realms.

Front Fencing

Garage storage and vehicle access

Gardens and landscaping

Siting and setbacks

Building height and form

Design Guidelines - Bush Residential 1

•

New development should complement the 1-2 storey building height and simple building forms of existing dwellings.

Buildings should not penetrate the indigenous tree canopy, where a canopy is present.

Buildings should be designed to follow the topography of the land, and minimise the need for cut and fill throughout the site.

New buildings at or near ridgelines should be designed and sited below the height of trees along the ridgeline.

Complement the building forms, natural materials such as mudbrick, earthy colour palette and siting of existing dwellings and articulate the front façades of buildings through the use of setbacks, recesses and windows.

Buildings should provide a pitched roof with prominent eaves.

•

•

•

•

•

•

Buildings should be set back a minimum of 4m from one side boundary to enable the planting of indigenous trees and understorey planting.

If more than one dwelling is proposed, provide sufficient separation between each dwelling to allow for the planting of indigenous/native trees and understorey vegetation.

Buildings should be sited to take into account the sharing of views corridors to the Diamond Creek, the Watsons Creek, the Yarra River, elevated ridgelines, vegetated areas and canopy trees.

•

•

•

Retain existing indigenous and native canopy trees and understorey vegetation and replant wherever possible. If this cannot be achieved, or a tree is considered appropriate for removal, the site should provide adequate space for offset planting of native trees that will grow to a mature height similar to the mature height of the tree to be removed.

Prepare a landscape plan to accompany all applications for new dwellings that utilise appropriate native species as identified in the Nillumbik Live Local Plant Local Guide.

Provide significant indigenous and native canopy trees occurring at a density of one to every 50-100m2 across the site.

The site area covered by buildings should not exceed 40 per cent. Provide at least 40% of the site as permeable surface.

•

•

•

•

•

Locate garages and carports behind the line of the front dwelling façade, ensuring that they are integrated with the design of the dwelling.

Minimise paving in front yards, including the driveway. Provide only one vehicular crossover per typical site frontage.

Limit the width of vehicle accessways and minimise hard paving within the front setback.

Provide landscaping and plantings to the soften the appearance of driveways.

•

•

•

•

•

Fencing should be set back from the front site boundary to allow for landscaping in front of the fence.

A front fence within 3 metres of a street should provide no or a low, open style up to 1.2m in height.

•

Draft Nillumbik Shire Neighbourhood Character Strategy

57

Character Elements	Design Responses

Zoom in Maps

Bush Residential 1 - Diamond Creek

Bush Residential 1 - Wattle Glen

58

Draft Nillumbik Shire Neighbourhood Character Strategy

Bush Residential 1 - North Warrandyte

Bush Residential 1 - Hurstbridge

Draft Nillumbik Shire Neighbourhood Character Strategy

59

Bush Residential 1 - Hurstbridge

Bush Residential 1 - Hurstbridge

60

Draft Nillumbik Shire Neighbourhood Character Strategy

Bush Residential 1 - St Andrews

Bush Residential 1 - Panton Hill

Draft Nillumbik Shire Neighbourhood Character Strategy

61

Bush Residential 1 - Research

Bush Residential 1 - Eltham

62

Draft Nillumbik Shire Neighbourhood Character Strategy

Bush Residential 1 - Eltham/Eltham North

Draft Nillumbik Shire Neighbourhood Character Strategy

63

Bush Residential 1 - Eltham/Research

64

Draft Nillumbik Shire Neighbourhood Character Strategy

Bush Residential 2

Character Attributes

7.4

Character Description

The Bush Residential 2 precinct is characterised by formal streetscapes and densely vegetated, landscaped dwellings. The architectural style of dwellings in this precinct includes a mix of Post-war and Modern styles predominantly constructed of brick with gabled roof forms. Dwellings are detached and feature generous front and side setbacks.

Dwellings are often obscured from view at street level due to the density of native and indigenous tree canopy and planting.

Dwellings are set within heavily vegetated lots, featuring a mix of native and non-native vegetation that often responds to the slope of the land. Private gardens feature

mulched areas retained by timber walls, natural stone walls or boulders. A lack of front fencing is notable and creates an openness to the streetscape.

Another distinguishing characteristic of this precinct is the formal road network, consisting of sealed roads and vehicle crossovers, drainage infrastructure and a single-sided footpath in some areas. The road layout follows a curving street pattern through the undulating landscape. Road cross-sections vary between narrow and standard.

Draft Nillumbik Shire Neighbourhood Character Strategy

65

Hurstbridge

Eltham North

Architectural Style, form and layout

- Mix of Post-war and Modern era styles with an occasional Contemporary development.

- Buildings are detached.

- Front façades are articulated with the use of balconies, porch entrances, windows and doorways.

- Roof forms are generally gabled and flat in contemporary developments.

Building materials

- Predominantly brick or render, and other contemporary materials.

Building heights

- 1 to 2 storey dwellings.

Roof styles

- Mix of low and high pitched (gable or hip) or flat roofs constructed of metal or tile.

Garages and carports

- Car parking is often located behind the facade of the dwelling or constructed on the side boundary.

Orientation and Siting

- Inconsistent siting and orientation of dwellings.

- Front setbacks are generous ranging between 7m to 9m and occasionally in excess of 9m.

- Dwellings are separated by spacious side setbacks, in excess of 4m.

Front Fencing

- Low, transparent front fencing up to 1m.

Gardens and Vegetation

- Formally landscaped gardens, comprising a mix of native and non-native plants, and remnant indigenous trees.

Public Realm

- Streets have a modified grid layout, with curved roadways with cul-de-sacs.

- Formal streets comprising sealed asphalt and a single concrete footpaths in some areas.

- Drainage usually consists of concrete rollover kerbs.

- Undulating to sloping topography.

Bush Residential 2

Legend

Nillumbik municipal boundary Road network

Bush Residential 2

St Andrews

CITY OF WHITTLESEA

Hurstbridge

Panton Hill

Yarrambat

Wattle Glen

Plenty

Greensborough

Diamond Creek

Eltham North

Research

CITY OF BANYULE

North Warrandyte

Eltham

CITY OF MANNINGHAM

0

2

4km

66

Draft Nillumbik Shire Neighbourhood Character Strategy

Neighbourhood Character Objectives

1.

To ensure new development reflects the preferred built form, characterised by predominantly 1-2 storey dwellings with varied architectural style including simple building forms and low-pitched roofs with prominent eaves.

To ensure new development reflects the existing neutral colour palette of natural materials.

To ensure new development sits within the existing indigenous and native tree canopy and enhances the bushy vistas, streetscapes and backdrops of the precinct.

To maintain and reinforce the predominant rhythm of dwelling spacing within the streetscape, characterised by intermittent views of the Diamond Creek, the Watsons Creek, the Yarra River, elevated ridgelines, vegetated areas and canopy trees afforded by undulating topography and spacious setbacks that enable substantial planting between each dwelling and the street.

To retain and reinforce the spaciousness and bushy setting of the streetscape, characterised by no or low, permeable front fences with views to front garden areas.

2.

3.

4.

5.

Draft Nillumbik Shire Neighbourhood Character Strategy

67

Diamond Creek

Hurstbridge

Eltham North

Wattle Glen

Preferred Character Statement

Large indigenous and native canopy trees and understorey planting in the public and private realm, combined with the undulating topography, ensures that dwellings do not visually dominate the streetscape, enhancing the bushy vistas, streetscapes and backdrops of the precinct.

View corridors through to the Diamond Creek, the Watsons Creek, the Yarra River, elevated ridgelines, vegetated areas and canopy trees are retained.

Dwellings sit within bushy garden settings featuring timber walls, natural stone walls or boulders.

New development positively responds to the predominantly low scale, one to two storey dwellings, using consistent materials and simple building forms.

Garages and carports are often located behind the line of the front dwelling façade and are unobtrusive and integrated with the design of the dwelling.

Absent, low or transparent, front fencing contributes to the bushy and informal transition between public and private realms.

Mostly sealed roads wind through the undulating landscape and are dominated by substantial indigenous and native trees, with a presence of footpaths on one side the street.

Front Fencing

Garage storage and vehicle access

Gardens and landscaping

Siting and setbacks

Building height and form

Design Guidelines - Bush Residential 2

•

New development should complement the 1-2 storey building height and simple building forms of existing dwellings.

Buildings should not penetrate the native and/or indigenous tree canopy, where a canopy is present.

Buildings should be designed to follow the topography of the land, and minimise the need for cut and fill throughout the site.

New buildings at or near ridgelines should be designed and sited below the height of trees along the ridgeline.

Complement the building forms, natural materials, muted colour palette and siting of existing dwellings and articulate the front façades of buildings through the use of setbacks, recesses and windows.

Buildings should provide a pitched roof with prominent eaves.

•

•

•

•

•

•

Buildings should be set back a minimum of 4m from one side boundary to enable the planting of indigenous and native trees and understorey planting.

If more than one dwelling is proposed, provide sufficient separation between each dwelling to allow for the planting of medium indigenous/native trees and understorey vegetation.

Buildings should be sited to take into account the sharing of views corridors to the Diamond Creek, the Watsons Creek, the Yarra River, elevated ridgelines, vegetated areas and canopy trees.

•

•

•

Retain existing indigenous and native canopy trees and understorey vegetation and replant wherever possible. If this cannot be achieved, or a tree is considered appropriate for removal, the site should provide adequate space for offset planting of native trees that will grow to a mature height similar to the mature height of the tree to be removed.

Prepare a landscape plan to accompany all applications for new dwellings that utilise appropriate native species as identified in the Nillumbik Live Local Plant Local Guide.

Provide large indigenous and native canopy trees occurring at a density of one to every 100m2 across the site.

The site area covered by buildings should not exceed 40 per cent. Provide at least 40% of the site as permeable surface.

•

•

•

•

•

Locate garages and carports behind the line of the front dwelling façade, ensuring that they are integrated with the design of the dwelling.

Minimise paving in front yards, including the driveway. Provide only one vehicular crossover per typical site frontage.

Where a side by side development occurs, space the vehicle crossovers to retain the existing rhythm of the street.

Provide landscaping and plantings to the soften the appearance of driveways.

•

•

•

•

•

Fencing should be set back from the front site boundary to allow for landscaping in front of the fence.

A front fence within 3 metres of a street should provide no or a low, open style up to 1.2m in height.

•

68

Draft Nillumbik Shire Neighbourhood Character Strategy

Character Elements	Design Responses

Zoom in Maps

Bush Residential 2 - Hurstbridge

Bush Residential 2 - Diamond Creek

Draft Nillumbik Shire Neighbourhood Character Strategy

69

Bush Residential 2 - Wattle Glen

Bush Residential 2 - Diamond Creek

70

Draft Nillumbik Shire Neighbourhood Character Strategy